Workbook 11.2: Security Countermeasure

Policies and Procedures

| Purpose: | To provide information about selected security countermeasures that require policies and procedures |
| --- | --- |

Section 1: Information about Perimeter Barriers

In addition to the standards for fences, there are standards that should be included in policies and procedures for perimeter barriers such as doors, windows, and other areas of the facility including but not limited to roofs, loading docks, public utilities, and air conditioning systems.

Standards for Doors

| External doors | Install external doors made of solid hardwood, metal, solid laminated core, or solid multi-ply construction |
| --- | --- |
|  | Strengthen solid softwood doors with a steel plate over the outer face of the door |
|  | Configure external doors to open outwards |
|  | Cover external door hinges to prevent pin removal |
|  | Ensure doors are close-fitting and equipped with suitable locks |
|  | Seal or protect letter receptacles and doors not in use |
|  | Install locking bars across the back of the door |
| Glazed doors | Strengthen by adding a steel a mesh grille |
| Double doors | Fasten with bolts attached to the first closing leaf of the door and add a security deadlock |
| Interconnecting doors | Apply the same reinforcement technique as for external doors |
| Internal doors | Keep the keys for the locks secure |
| Emergency exit doors | Ensure compliance with local area fire regulations |
| Door frames | Strengthen the frame and its attachment to the building structure |
| Door bolts | Use door bolts in conjunction with security locks and fitted in pairs |
|  | Ensure that bolts cannot be opened from outside the door |
|  | Ensure that the bolt engages into the floor fully and the hole in the floor is kept free from obstruction |

Standards for Windows

| Windows | Brick up or otherwise secure all accessible nonessential windows |
| --- | --- |
|  | Install secure fittings to all basements, ground floor, and other windows which are readily accessible |
|  | Examine window catches and replace defective catches |
|  | Consider providing intruder alarms for protection |
|  | Identify responsibility for securing windows with potential access from ground floor level or reasonable climbing heights |
|  | Ensure ground floor windows and those that are easily accessible to entry have locking handles or opening restrictors |
|  | Consider installing bars, grilles, or shutters where necessary |
|  | Consider installing intruder detection sensors |
|  | Apply double glazing to provide additional protection against surreptitious or forcible attack |

Standards for Other Areas

| Grilles and shutters | Use expanding grille gates, roller grilles, or roller shutters in doorways, passages, or other openings |
| --- | --- |
| Roofs | Block access to facility rooftops by erecting suitable barriers, inserting grilles, and other potential entry points |
|  | Install intruder alarms to detect unauthorized access |
|  | Incorporate suitable measures to protect access to a building from attic or roof space |
| Skylights, fanlights, and roof lights | Replace the glass with security glazing material |
|  | Fit the lights with bars or grilles |
|  | Screw the frames in place and cover the glass with steel mesh |
|  | Install an intruder detection system |
| Downpipes | Restrict access by boxing in the pipes or by treating them with anticlimb devices |
| Sunken outside areas | Secure steel grilles or steel mesh screens from below |
|  | Fit vents into sunken areas or vents emerging at street level with internal steel grilles |
|  | Install padlocked crossbars fitted on the inside to prevent access through service tunnels |
|  | Install an intruder detection system |
| Loading docks | During business hours, use a custodian or electronically operated shutters so that the bay doors only open when an incoming vehicle is identified |
|  | After business hours, close roller shutters or sliding shutters that secure from the inside |
| Public utilities | Install grating and intruder detection systems to block usage of cables and pipes entering the building |
| Air conditioning and other systems | Restrict access to provide adequate protection from sabotage |
| Multi-occupancy buildings | Ensure other tenant organizations secure their portion of the building at the same level as you |
|  | Ensure that the security measures applied to your organization consider the existing tenant security levels |

Section 2: Information about Closed-Circuit Television

Another type of policy and procedure to develop is for the use of closed-circuit television.

Benefits

Saves manpower, especially when used in conjunction with an intruder detection system and automated access control system

Supplements and extends, making an existing security system more effective

Enhances the effectiveness of perimeter security particularly if used to verify the alarms signaled by perimeter intruder detection systems

Functions

Assists with security personnel effectiveness

Can be used to direct responses and assist in the control of incidents

Deters intruders

Assists in post-event analysis and incident investigation

Assists with entry control

Provides general operational information to aid in running the premises

Provides site monitoring at night

Can be fitted with video detection capability to monitor the external areas to the facility

Can be used as a management tool for alarm verification

Image Requirements

Image Standards

Closed-circuit television cameras should be an integral part of the physical protection system and deployed in areas where they achieve the following standards:

Position closed-circuit television cameras so that you are able to monitor the main entrance for entry and exit surveillance (identification).

Use cameras at each external access control point and in conjunction with an intercom, if direct viewing of visitors is not possible (identification).

Support all access-controlled perimeter barriers with the use of intercom and closed-circuit television cameras (recognition).

Ensure closed-circuit television cameras provide surveillance of the facility perimeters with no gaps (recognition).

Install closed-circuit television cameras so that you can view vehicle parking areas (recognition).

Place closed-circuit television cameras to monitor the entrance and exit from critical areas and secondary areas (identification), including:

Facility back-up power systems

Security control center

Heating and ventilation supply

Switching rooms and build rooms (power supply rooms)

Confirm that closed-circuit television cameras provide internal area surveillance (recognition).

Ensure that the lighting is adequate and compatible with closed-circuit television cameras to ensure that they provide clear images (detection, recognition, identification).

Verify that closed-circuit television systems are installed with video motion detection, where necessary (detection, recognition, identification).

Monitor all installed and properly maintained closed-circuit television systems on a 24-hour a day, 7 days a week basis.

Ensure that an appropriately trained security force is available to respond to any identified alarms or incidents.

The system should be capable of being remotely monitored and recording should preferably be digital and be retained for a minimum of 30 days (recognition and identification).

Register the systems, where applicable, with the local law enforcement (detection, recognition, identification).

Digital recording of closed-circuit television cameras is recommended and should meet the specification details established by the chief of security.

Specific requirements will depend on the type of digital recorder to be installed, the cost appraisal for the number of cameras to be recorded on opening and the number of cameras likely to be used in the future.

The digital recording system should be controlled by the chief of security.

Section 3: Information about Lock and Key Controls

Use the following guidelines when developing policies and procedures related to lock and key controls.

Standards

Issue keys based on authorized employee and contractor identification system

Keep the number of keys issued for any lock to a minimum

Maintain a record (master key register developed by the local security force) showing the following:

The identifying features of each key such as the type registered, key number, and number of the duplicate

The identifying details of persons allowed access to each key

The date the working key (but not the duplicates) was signed out to the custodian

Check the keys in at the end of each working day when used to directly protect sensitive environments; place unused keys in locked key boxes within the security control center

Ensure that keys are not accessible to persons who do not have authorized access to the material or to the room (area) that the lock protects

Do not remove security keys from the facility without the specific authority of the lead security officer

Treat in-use security keys with the same value and sensitivity as the material and environment they protect

Store, protect, and handle the keys accordingly

Conduct periodic checks so to account for all keys

Change keys and combinations when any of the following events occur:

Loss or compromise of keys

Suspected loss or compromise of keys

Termination of employment of any key holder

On a regular basis (for example, every 90 days)

Spare Keys

Place spare keys to security locks in approved security containers by a designated member of staff

Do not hold spare keys in the same container as the working key

Issue a spare key only to persons with authorized access to the area or material the lock protects and only with documentation proving that the working key has been mislaid or lost

Record the details of any spare keys that have been issued

Supply additional keys only on the written authority of the lead security officer

Key Labeling

Label keys to facilitate daily issue with clear markings and designated hook on key panel

Do not identify the specific item the key opens:

Gate

Door

Container

Check key rings or fobs frequently to ensure that keys cannot become detached. Note: a key fob is a type of security token. It is a small hardware device with built-in authentication mechanisms to control access to network services and information.